### Course: AKM-201

### MODULE: DIGITAL CLAIM PEGGING (GPS)

Status: Level 2 (Technical) | XP Reward: 800 XP | Badge: The Surveyor

### 👋 Welcome to Level 2!

You know what to look for (Geology). You know where to look (Terrain). Now, you need to secure it. In the past, pegging a mine required hiring an expensive surveyor with a theodolite. Today, you have a precision survey tool in your pocket: Your Smartphone.

Your Mission: Learn to use the NMIH App’s GPS mode to "Virtually Peg" a site and create a digital boundary file (.gpx) that the Ministry of Mines can accept.

### UNIT 1: THE EVOLUTION OF PEGGING

From Metal Stakes to Digital Polygons.

1. The Old Way (Physical Pegging)

Traditionally, a miner marks their claim by hammering metal stakes into the ground at four corners.

- The Risk: Physical pegs get stolen, knocked over by cattle, or hidden by tall grass.

- The Cost: Hiring a surveyor to measure these points costs hundreds of dollars.

2. The Smart Way (Geofencing)

A "Digital Peg" is a precise GPS Coordinate (Latitude/Longitude) stored in the NMIH Cloud.

- Immutable: It cannot be moved or stolen.

- The Polygon: Instead of just 4 points, we create a "Shapefile"—a digital drawing of your land that calculates the exact area size instantly.

### UNIT 2: THE "WALK" (Field Technique)

How to capture a professional boundary.

1. The Setup

Mining claims in Zimbabwe are strict squares or rectangles (e.g., Gold Reef claims are often 200m x 200m).

- Step 1: Go to Corner A (Your Starting Post).

- Step 2: Open the NMIH App > Select "Start Survey."

- Step 3: WAIT. Stand still for 30 seconds.

  - Why? This allows the GPS to "triangulate" with more satellites, bringing accuracy down from 10 meters to 3 meters.

2. The Trace

Walk steadily to Corner B, then C, then D.

- The Track Log: The App draws a line following your feet.

- Obstacles: If there is a large termite mound or bush, pause the track, walk around it, and resume on the other side. A wobbly line looks unprofessional to the Ministry.

3. Closing the Loop

You must return exactly to Corner A to "Close the Polygon."

- Result: The App immediately displays the Area: "You have enclosed 9.8 Hectares."

### UNIT 3: THE FORMATS (Talking to Government)

Speaking the language of the Ministry of Mines.

1. The Output

The Ministry uses GIS software. They cannot use a screenshot; they need raw data.

- GPX (GPS Exchange Format): This is the raw code of your movement.

- KML (Keyhole Markup Language): This is the file format used by Google Earth.

2. The Deliverable

When you finish your survey, the NMIH App creates a .KML file. You can email this file directly to a Mining Commissioner or Investor. When they open it, they will fly straight to your claim on Google Earth and see your exact blue boundary line.

### 🏆 FIELD CHALLENGE (The Gig)

Complete this task to earn your "Surveyor" Badge.

Task: The Virtual Peg

- Go to an open space (a field, park, or your garden).

- Open the NMIH App > Select "New Survey."

- Walk a perfect square (at least 20 meters x 20 meters).

- Drop a "Digital Pin" at all 4 corners.

- Save & Upload: The App must show a "Closed Loop" (Polygon) and calculate the area.

### 📝 QUIZ: THE DIGITAL SURVEYOR

Module: AKM-201 | Time Limit: 5 Mins | Pass Mark: 4/5

Q1: Why do we stand still for 30 seconds at the starting corner before recording?

A) To rest.

B) To let the GPS satellites "lock on" and improve accuracy.

C) To take a selfie.

D) To wait for the internet.

Q2: What is a "Shapefile" or "Polygon" in digital mining?

A) A star shape.

B) A digital drawing that connects your coordinates to define the area of your claim.

C) A type of rock.

D) A password.

Q3: Which file format allows an investor or the Ministry to view your claim on Google Earth?

A) .MP3

B) .KML

C) .JPG

D) .DOC

Q4: If you walk around a large tree while recording your boundary, what happens to the line?

A) The line stays straight.

B) The line will curve around the tree, making the boundary look unprofessional.

C) The App deletes the data.

D) You get a bonus.

Q5: A standard Gold Reef claim in Zimbabwe is often pegged as a block of roughly:

A) 10 Hectares (e.g., 200m x 500m or similar).

B) 1 Meter x 1 Meter.

C) The size of a football.

D) 1000 Kilometers.

SUBMIT ANSWERS & CLAIM 800 XP