# 📱Course: AKM-303

## MODULE: STAYING ALIVE (Mine Safety & SHEQ)

Status: Level 3 (Compliance) | XP Reward: 1000 XP | Badge: The Safety Officer

### 👋 Welcome to the Danger Zone!

Mining is not just about finding gold; it is about coming home alive. Every year, miners die from collapsing shafts, bad air ("gassing"), or chemical poisoning.

Your Mission: Learn the "Zero Harm" principles. You will learn how to secure your shaft, handle deadly chemicals like Cyanide/Mercury, and prepare for emergencies.

### UNIT 1: SHAFT SAFETY (Ground Control)

How to stop the ground from killing you.

1. The "Collar"

The most dangerous part of a mine is the entrance (the collar).

- Rule: You must concrete or timber the top 3 meters of any shaft. Rain makes the surface soil loose—if the collar collapses, it traps everyone inside.

2. Ventilation (The Air You Breathe)

- The Killer: Carbon Monoxide (from blasting) and "Bad Air" (low oxygen).

- The Test: Never enter a deep shaft without testing the air.

  - Old School: Lower a burning candle. If it goes out, there is no oxygen. DO NOT ENTER.

  - Smart School: Use a Winch or Fan to blow fresh air down the shaft for 30 minutes before every shift.

3. Support (Timbering)

- Rule: "Loose ground must be supported." If you see cracks in the rock roof, put in a timber prop immediately.

### UNIT 2: THE CHEMICALS (Cyanide & Mercury)

The Silent Killers.

1. Mercury (The Gold Magnet)

- The Danger: Mercury attacks your brain. Never touch it with bare hands. Never burn amalgam (gold-mercury mix) in a hut or kitchen. The vapor is deadly.

- The Solution: Always use a Retort (a closed burner) to catch the mercury vapor so you can reuse it.

2. Cyanide

- The Rule: Cyanide dissolves gold, but it also stops human breathing instantly.

- Storage: Keep cyanide locked in a dry room. Never store it near food.

- The Antidote: Every cyanide site must have a "Cyanide Antidote Kit" nearby and someone trained to use it.

### UNIT 3: EMERGENCY RESPONSE (The Plan)

What to do when things go wrong.

1. The Whistle

Every mine must have a loud alarm (whistle, bell, or siren). When it sounds, everyone runs to the Assembly Point.

2. The PPE (Personal Protective Equipment)

No boots, no helmet = No work.

- Helmet: Stops falling rocks.

- Gumboots: Protects feet from chemical burns and sharp rocks.

- Dust Mask: Stops "Silicosis" (dust coating your lungs).

### 🏆 FIELD CHALLENGE (The Gig)

Complete this task to earn your "Safety Officer" Badge.

Task: The Safety Audit

- Go to your mine shaft or processing area.

- Check 1: Is everyone wearing a helmet?

- Check 2: Is the shaft entrance secured (fenced or collared)?

- Check 3: Identify the "Assembly Point" (safe zone).

- Photo: Take a photo of a safety sign or someone wearing correct PPE.

- Tag it: "Safety Audit Passed" or "Risk Identified."

[BUTTON: SUBMIT AUDIT]

### 📝 QUIZ: ZERO HARM

Module: AKM-303 | Time Limit: 5 Mins | Pass Mark: 4/5

Q1: What is the most common cause of "Gassing" accidents in deep shafts?

A) Eating bad food.

B) Lack of Oxygen or presence of Carbon Monoxide (often after blasting).

C) Too much sunlight.

D) Ghosts.

Q2: Why must you concrete or timber the "Collar" (top part) of a mine shaft?

A) To make it look nice.

B) To prevent rain from washing loose surface soil into the shaft, which causes a collapse.

C) To stop thieves.

D) To keep the shaft warm.

Q3: Mercury is dangerous to the brain. What tool should you use when burning mercury-gold amalgam to stay safe?

A) An open spoon over a fire.

B) A Retort (a sealed distillation device).

C) A frying pan.

D) A microwave.

Q4: If you are using Cyanide to leach gold, where should you store the chemical?

A) In the kitchen with the salt.

B) In a locked, dry, well-ventilated room away from food and water sources.

C) Under your bed.

D) In an open bucket outside.

Q5: What is "Silicosis"?

A) A lung disease caused by breathing in fine rock dust over many years.

B) A type of gold.

C) A brand of shovel.

D) A safety helmet.

SUBMIT ANSWERS & CLAIM 1000 XP